2021-07

ASSA ABLOY

OH1042S Overhead Sectional Speed Door

Tender Specification Text

# ASSA ABLOY OH1042S Overhead Sectional Speed Door

Model

ASSA ABLOY OH1042S fully balanced sectional speed door with an operating speed of ≈1,0 m/s up and 0,7 m/s down.

Incorporating insulated door panels manufactured with a broken cold bridge to provide minimal thermal transmittance and reducing risk of condensation and ice formation. Optimized panel joint for even load distribution minimizing impact stress on panel hinges to prevent panel delamination and cracks. Life time expectations door: 200.000 door cycles or 10 years, if service/replacement program is performed. Facilitates a tight seal throughout the life cycle of the door. Secured By Design certified. Diamond pattern exterior panel finish.

Dimensions

Clear opening width	2.000 mm - 5.000 mm

Clear opening height	2.750 mm - 5.000 mm

Standard equipment

- Fully balanced, powder coated pre tensioned springs, designed for a minimum 20000 operating cycles, mounted on a 35 mm shaft with extra side bearings.

- Flat maintenance free hinges made of composite material with a stainless steel pin.

- Flat roller brackets with double rollers, no protruding parts on the door leaf.

- EPDM perimeter and invisible horizontal joint seals.

- Finger pinch protection to horizontal panel joints.

Performance

To comply with European Standards

EN 12424 	Wind resistance

for doors having insulated panels only: Class 3 (DLW ≤ 4250) Class 2 (4250 ＜DLW)

for doors having framed sections in 2 and 3: Class 3 (DLW ≤ 3650) Class 2 (3650 ＜DLW ≤ 4550) wider doors (4550 ＜DLW ≤ 5000) possible if framed sections are only in sections number 4 and up.

EN 12425 	Water penetration class 3 (70 pa resistance).

EN 12426 	Air permeability class 3.

EN 12428 	Thermal transmittance 1,0 W/(m²∙K).

DIN 4102 	Fire performance class B2

EN 13241 	CE marked with manufacturer’s declaration of conformity.

ISO 9001	Quality management

ISO 14001	Environmental management

Compliant with L2A Building Regulations.

Door panels

- 545 mm deep, 42 mm thick process laminated steel panels with internal steel bands positioned to ensure proper fixation of hinges and roller brackets throughout the products life span. CFC free insulation giving an optimum panel u value of 0.4 W/(m²∙K).

- Panel design incorporates an external diamond profile steel skin for additional strength and aesthetic appearance with glossy finish at the section joints. Bottom profile equipped with oil resistant EPDM triple lip rubber seal to minimize leakage.

- Finish as delivered

- Internal, monolithic, pre coated polyester to RAL 9002.

- External pre coated polyester from ASSA ABLOY standard range

- RAL 1021 Rape Yellow, RAL 3000 Flame Red, RAL 5003 Sapphire Blue, RAL 5010 Gentian Blue, RAL 6005 Moss Green, RAL 7016 Anthracite Grey, RAL 7021 Black Grey, RAL 7024 Graphite Grey, RAL 8017 Chocolate Brown, RAL 9002 White, RAL 9005 Jet Black, RAL 9006 Metallic Silver, RAL 9007 Silver Grey & RAL 9010 Pure White

- Or Factory applied paint finish to the external face of the door panels in RAL ……..

- All rubber sealants from EPDM rubber, temp. resistant -30/+70 Celsius

- All steel components electro galvanized

- All parts should be system specific, designed for purpose

Tracks

2 piece galvanized tracks complete with 25 mm thermal movement provision.

Standard Lift (SL) 	head room 510 mm

High Lift (HL) 	head room HL+370 mm

Vertical Lift (VL) 	head room DLH + 560 mm

Operator

- Operating voltage: 230V AC +/- 10%, 50 HZ (internal supply 3-phase 230V AC) 0,55 kW.

- Operating factor: ED = 30%, S3 10 min, non-continuous operation

- (Motor operation 3 min, pause 7 min, direct-inverting)

- Torque: max 80 Nm, depending on drum type and speed.

- Fuse: 10 A

- Brake: Electro-magnetic spring-loaded brake

- Speed monitor: electronic monitoring

- Freewheel / clutch: mechanical engagement/disengagement.

- Limit switch: electronic via encoder.

- Gears: worm gear, not self-locking, max 120 RPM on door shaft.

- Shaft: ø35 mm tube or solid

- No special wall reinforcement necessary

- Temperature range: -20°C to +55°C. At low temperatures the first few cycles may be run with reduced speed to prolong the operator’s lifetime. Can be equipped with a heater for a working range down to -30°C.

- Soft stop, soft start FC controlled

- Connection using “CEE plug” or lockable mains isolator

- Maximum door weight 300 kg

Control unit 950 with Light Grid

- 1 x 230 V +/- 10% supply, 18 W.

- Dimensions: 180 x 380 x 120 mm (w x h x d) including control unit with standard actuator UP-STOP-DOWN and pulse control incl. self- monitoring of safety edge (light grid).

- Automatic closing, the door closes automatically after set period (2-932s).

- Counter/error code display and servicing info display in top of control cabinet.

- Protection class IP 44 with connector, IP 65 when wired.

- Disengage device to be used in case of power failure is optional.

Safety device(s)

- Spring break device

- Safety factor six door cables

- Stop and return photocell light grid

Locking mechanism

Interlocked sliding bolt with lock bolt switch

Warranty

1 YEAR / 2 YEAR / 3 YEAR / 4 YEAR / 5 YEAR extended manufacturer’s warranty available to secure optimum performance and longevity.

Please consult ASSA ABLOY Entrance Systems technical literature for further details and the quotation for project specific notes.

OPTIONS

Windows

- Sandwich glazing, double glazed, rectangular, 604 x 292 mm (DARP)

- Sandwich glazing, triple glazed, rectangular, 604 x 292 mm (TARP)

- Sandwich glazing, double glazed, oval, 610 x 292 mm (DAOP)

- Sandwich glazing, double glazed, rectangular, 578,5 x 268,5 mm (ALRB)

- Sandwich glazing, double glazed, rectangular, 578,5 x 146,5 mm (ALBS)

- 545 mm Aluminium frame bar section with, acrylic sheets, 2 x 2,5 mm, total thickness 27 mm, single sealed

- 545 mm Aluminium frame bar section with, scratch resistant acrylic glass, 2 x 2,5 mm, total thickness 27 mm, single sealed

- 545 mm Aluminium frame bar section with, acrylic glass 2 x 2,5 mm, total thickness 27 mm, double sealed, with anti-fog granulate in the bars.

- 545 mm Aluminium frame bar section with, scratch resistant acrylic glass 2 x 2,5 mm, total thickness 27 mm, double sealed with anti-fog granulate in the bars.

- 545 mm Aluminium frame bar section with, single glazing 2,5 mm acrylic

- 545 mm Aluminium frame bar section with, scratch resistant single glazing 2,5 mm acrylic

- 545 mm Aluminium frame bar section with, single glazing 4 mm hardened glass (On request)

- 545 mm Aluminium frame bar section with, triple glazing – scratch resistant

- 545 mm Aluminium frame bar section with, triple glazing – not scratch resistant

Door locking device

- Second sliding shoot bolt

- Lock with profile cylinder lockable on one or both sides

- Optional burglar proof class 2-RC1+RC2

Control unit Options

- Radio remote control

- “Open” (if open button is used during closing process - door immediately stops and opens completely)

- Pull switch for OPEN / CLOSE

- External push-button, key switch

- “One-button function” - opens or closes door, depending on actual status (e.g. radio, pull switch)

- additional safety (e.g. photo cell, radar) – door immediately opens completely

- D-Module (D1-D6)

- Photo cell, typically as a reset for automatic closing, some distance from the door.

- Reduced opening height

- Radar

- Magnetic loop

- Warning lights

- Traffic lights, on request.

- Interlocking door function, on request.

- Door/bridge interlock

- French Warning lights

- LED on door

- Indication open and/or closed door.

- Additional 24 V DC, 430 mA

- Light grid as reset for automatic closing, instead of additional photocell, optional.

Note! Lights and position indications uses the two available outputs, combinations are limited, expansions/combinations on request.

D-Module (automation module)

- D1 functions

- red lamps, inclusive, blinking continuously when door is moving

- red blinking light before “Automatic close”

- constant red light with door closed available

- Start time for blinking lights before closure can be set

- D3 - function incl.

- D2 functions

- Magnetic loop, inclusive, with various functions, e.g.

- “Safety and checkback signal”

- “Open and checkback signal”

- “Safety and open”

- “Open”

- A DIP switch can also be chosen to provide confirmation as additional safety

- D3 - function incl.

- D3 - functions

- Interlocking door function or mutual interlocking between two doors

- Door/bridge interlocking

- Incl. 24 V AC max. 400 mA for additional units

- Relay for “Door open” or “Door closed”

- D4 functions

- Combination of D1 and D2

- red lamps (inclusive) blinking continuously when door is moving

- red blinking light (inclusive) before “Automatic close”

- constant red light with door closed available

- Start time for blinking lights before closure can be set

- A DIP switch can also be chosen to provide confirmation as additional safety

- D3 - function incl.

- magnetic loop (inclusive) with various functions, e.g.

- “Safety and checkback signal”

- “Open and checkback signal”

- “Safety and open”

- “Open”

Note! Lights and position indications uses the two available outputs, combinations are limited, expansions/combinations on request.

- D5 functions

- Directional sensor traffic light (including red/green)

- Control of oncoming traffic in the doorway

- using “Open interior” or “Open exterior” to control oncoming traffic

- Oncoming traffic sees alternately red or green light, depending on which “Open command” is given first

- D3 - functions

- D6 functions

- Directional sensor traffic light with magnetic loop activation (inclusive), of “Open exterior”

- Control of oncoming traffic in the doorway

- using “Open interior” or “Open exterior” to control oncoming traffic

- Oncoming traffic sees alternately red or green light, depending on which “Open command” is given first

- D3 - functions